Sköntorp Bus Display

Technical description of the live arrival feature for SL bus 168 toward Södersjukhuset

Version of 25 October 2026  ·  Status: PHP service, web page and ESP32 display in operation; battery test running

# 1. Purpose and scope

The system tells you when SL bus 168 toward Södersjukhuset will reach the stop Sköntorp, and where that bus is right now, so you can decide when to walk to the stop. It combines SL's departure data with real-time vehicle data from Trafiklab. It consists of a PHP service on an ordinary web host, a web page that presents the service's output, and a battery-powered ESP32 touch display for the front door that uses the same service.

Design goals:

- Show the next bus's remaining time and where it is (for example "Årsta torg [bus] Årstaskolan", "2 Hpl kvar").

- Keep the current bus in focus until it has really passed, also when it is early or late.

- Hide buses in the other direction, and buses that have already passed Sköntorp.

- Keep API keys on the server, never in the page or in the gadget.

- Keep the reply small and the layout fixed, so that a microcontroller can use it.

- Run on a battery for as long as possible: the gadget sleeps and wakes on touch.

This document describes where the information comes from, how it is processed, how well it was found to work, and what is still unverified.

# 2. Architecture

Figure 1. Data flow from the three SL/Trafiklab sources through bus.php to the clients.

| Component | Runs on | Role |
| --- | --- | --- |
| bus.php, bus_config.php | Your web host (PHP 8.3 with cURL) | Fetches the feeds, decodes and filters them, merges live and timetable buses, returns JSON. The config file holds the key, the secret and the stop data. |
| bus.html | Browser | Presents the JSON: next bus, countdown, position, stops left, list of three buses. |
| bus_display.ino, config.h | ESP32-C6 board (Arduino IDE) | The gadget: sleeps, wakes on touch, fetches the JSON, shows it for one minute. config.h holds Wi-Fi password, address and secret. |
| stop_display.html, sl_proxy.php | Browser (and web host) | Separate copy of the stop display for both directions, built only on the SL departures API. The proxy is needed only if the browser blocks the API call (CORS). |
| bus_log.py | Mac (Python, virtualenv) | Logs predictions from both sources every 30 s, records manual arrival marks, reports prediction errors. |
| check_trip.py, make_route_stops.py, analyze_gps.py | Mac (Python) | One-time helpers: look up trips in the static timetable, print LAST_STOP_IDS and ROUTE_STOPS, relate GPS distance to arrival time. |

# 3. Data sources

## 3.1 SL Transport API (departures)

Endpoint: https://transport.integration.sl.se/v1/sites/1512/departures?direction=2. Site 1512 is Sköntorp. Direction 2 is Södersjukhuset, direction 1 is Östbergahöjden. No API key is needed.

Fields used: line.designation, destination, direction_code, expected, scheduled and display. Times are Stockholm local time without an offset, so they are parsed as Europe/Stockholm.

Role in the system: the timetable fallback for buses that are not (yet) in the live feed. Behaviour seen in the measurements: in the first two measured buses the predicted time was identical at all three checkpoints (it behaved like a fixed timetable time); in the later ones it changed somewhat. It also lists a bus until it has departed.

## 3.2 Trafiklab GTFS Regional Realtime (SL)

Feeds: https://opendata.samtrafiken.se/gtfs-rt/sl/TripUpdates.pb and .../VehiclePositions.pb, with the Trafiklab key as a parameter. The format is Protocol Buffers (GTFS-Realtime), sent gzip-compressed; requests must accept gzip.

- TripUpdates: for each running trip, a list of the upcoming stops with predicted arrival time and delay. Used for the arrival time at Sköntorp, for the direction guard (the last stop of the list), for the feed's next stop, and for cancellation flags.

- VehiclePositions: latitude, longitude, bearing and speed per trip. Used for the position text and for detecting that a bus has passed.

Findings from debug output: SL leaves route_id and direction_id empty in the trip descriptors, and the stop fields of VehiclePositions (stop_id, current_status, current_stop_sequence) are empty, so neither route nor "bus is at stop X" can be read from the feed. Bearing and speed are provided. After a bus has passed Sköntorp, the feed still lists the Sköntorp stop for at least about two minutes.

Timestamps. SL fills in the three timestamps that GTFS-Realtime defines: the time the feed was produced, the time each prediction was calculated, and the time each GPS position was recorded. In one debug sample the feed was 11 seconds old, the prediction 27 seconds and the GPS position 3 seconds. The positions are therefore fresh, and the predictions are the older part. A timestamp says how old data is; it does not make a prediction more accurate.

Trafiklab applies usage limits per key. The free Bronze level was quoted during development as 30,000 realtime calls and 50 static downloads per month; these figures are from memory and should be checked on Trafiklab's site.

## 3.3 Trafiklab GTFS Regional Static (SL)

sl.zip with routes.txt, trips.txt, stops.txt and stop_times.txt. It uses its own key and is only read offline by the Python helpers, never by bus.php. It provides stop coordinates and names, the stop order of every trip, and the end stops. Trip IDs change from day to day, stop IDs are stable.

## 3.4 Identifiers used

| Identifier | Meaning |
| --- | --- |
| SL site 1512 | Sköntorp in the SL departures API |
| direction 2 / 1 | SL API direction codes: 2 = Södersjukhuset, 1 = Östbergahöjden |
| GTFS stop 9022001013119001 | Sköntorp stop point of line 168. Trips in both directions call at this stop ID. |
| GTFS stop 9022001010406003 | Södersjukhuset, final stop of the 117 trips toward Södersjukhuset (LAST_STOP_IDS) |
| GTFS stop 9022001010406002 | Södersjukhuset stop where the opposite-direction trips start |
| Sköntorp coordinates | 59.30117, 18.05226 (from stops.txt) |
| ROUTE_STOPS | Eleven stops from Julitavägen to Sköntorp, from make_route_stops.py, covering the whole route before Sköntorp |

# 4. Processing in bus.php

## 4.1 One request, step by step

- Check the secret in ?t= with a constant-time comparison. A missing SECRET in the config makes the service refuse to run.

- If a cached reply younger than 10 seconds exists for the same mode (with or without gps), return it. debug bypasses the cache.

- Fetch and decode TripUpdates, and take the time from a clock that is checked against Trafiklab's (4.8); keep upcoming arrivals at Sköntorp (4.2 to 4.4).

- Fetch the SL departures for direction 2 and keep line 168 toward Södersjukhuset (4.5).

- Merge live and timetable buses (4.5).

- With gps=1: fetch VehiclePositions for the first live buses, attach position and the "where" description (4.6), and hide buses that have passed (4.7).

- Return at most four buses as JSON and store the reply in the cache.

## 4.2 Selecting the right trips (offline, once)

The first attempt picked trips by the text of their headsign, but the headsign did not contain the expected text and no trips were found. The working method uses the stop order instead: among the trips of route 168, a trip is a target trip if it calls at Sköntorp and later at a stop whose name contains "Södersjukhuset". This gave 117 trips and one stop ID for Sköntorp. The result is stored in bus_cache.json and used by bus_log.py.

## 4.3 Direction guard

The Sköntorp stop ID is used by trips in both directions: a trip that starts at Södersjukhuset, calls at Sköntorp and ends at Julitavägen was found in the live feed and showed up as a phantom bus. Since the feed has neither route nor direction, the guard uses the last stop of the trip's remaining stop list. A trip is kept only if that last stop is in LAST_STOP_IDS (the end stop of the target trips, 9022001010406003). If the list has been cut down to Sköntorp alone, the trip is kept, since the direction cannot be told. The IDs are printed by check_trip.py from the static timetable.

## 4.4 Decoding and arrival times

- Protobuf reader. A small reader of the wire format (varints, length-delimited fields, 32-bit floats) written in plain PHP, so that no Composer packages are needed. Field numbers follow the GTFS-Realtime specification: for example FeedEntity.trip_update = 3, TripUpdate.stop_time_update = 2, StopTimeUpdate.stop_id = 4, StopTimeEvent.time = 2 and delay = 1. Negative delays arrive as 10-byte varints and are read as signed 64-bit numbers.

- Cancelled trips and skipped stops (schedule_relationship 3 on the trip, 1 on the stop) are dropped. Other flags are shown in debug output.

- Arrival time is the stop's arrival.time, or the departure.time if no arrival time is given. min = (arrival − now) / 60, rounded to one decimal.

- Next stop. The first stop of a trip's remaining list is the feed's own "next stop" and is used in 4.6.

- Late buses stay listed. A live bus remains in the list for up to LIVE_GRACE_S = 300 s after its predicted time, until the feed drops it or the GPS shows that it has passed.

## 4.5 Merging live and timetable buses

Live buses come first. For the timetable, the SL departures are filtered on line 168, direction_code 2 and a destination containing "Södersjukhuset"; the time is expected, or scheduled if that is missing. A timetable entry is added only if no live bus lies within MATCH_SECONDS = 480 s of it, because such an entry is taken to be the same bus. The result is sorted by time. This gives a live prediction for the buses that are on the road and timetable times for the later ones.

## 4.6 Where is the bus?

ROUTE_STOPS is the ordered list of the stops of the route (id, name, latitude, longitude), ending with Sköntorp. The list must cover the whole route; with a short list, a bus before the first listed stop gets a wrong text.

- Compute the great-circle (haversine) distance from the bus to every stop in the list. Take the nearest stop (call it i).

- If the nearest stop is farther than FAR_M = 2000 m, give no text.

- If the distance is at most AT_STOP_M = 120 m, the bus is at stop i; stops left = number of stops from i to Sköntorp.

- Otherwise place the bus on the stretch of the route it is closest to: for each pair of consecutive stops, compute the distance from the bus to the straight line between them. The bus is between the two stops of the closest stretch, and stops left counts from the stop it is heading to, Sköntorp included.

- If the feed's next stop (4.4) is one of the listed stops, stretches that lie behind it are ruled out, because the bus has already passed them.

- If the closest stretch is the first one and the bus is before its start, the text is "near" the first stop.

Why not "nearest stop and its nearer neighbour". The first version decided the side with the nearer of the two neighbouring stops. That fails at bends. On the route, Årsta torg is a bend: Årstafältet lies 733 m to the west, but Årstaskolan only about 380 m away to the north-east. A bus 120 to 230 m before Årsta torg is closer to Årstaskolan than to Årstafältet, so the page said "between Årsta torg and Årstaskolan" while the SL app correctly showed "between Årstafältet and Årsta torg" (reported twice). The error was reproduced with the real stop list and disappeared with the stretch method.

The page and the gadget turn this into "Årsta torg [bus]" (at the stop) or "Årsta torg [bus] Årstaskolan" (between two stops), plus "2 Hpl kvar" or "Nästa Hpl är din". The straight line between two stops only approximates the street, so on a winding stretch, such as the start of the route near Terrassen and Slättåkragränd, the text can still be wrong. Comparison with the SL app gave accurate positions otherwise.

## 4.7 Has the bus passed Sköntorp?

Sköntorp is a dead end: after the turn the bus drives back past the stops it has just served, and the feed keeps the Sköntorp stop for a while. Without extra logic the page would show the returning bus as "Nu, 2 stops left". The service therefore decides from the GPS, using a small state file in the server's temporary folder (per trip: closest distance seen, and whether the bus has been at Sköntorp; entries older than 3 hours are removed). A bus is hidden when one of these holds:

| Rule | Condition | Reason shown in debug |
| --- | --- | --- |
| Left the stop | It has been within 120 m of Sköntorp and is now farther away | has left the stop |
| Moving away | It is overdue by 30 s or more and its distance exceeds the closest distance seen by more than MOVING_AWAY_M = 80 m | moving away |
| Heading away | It is overdue, more than 150 m away, moving faster than 1 m/s, and its bearing differs by more than 110° from the direction to the stop | heading away |

A bus that is at Sköntorp is always shown, so that the position text appears before the next bus takes over. "Overdue" is computed from the min value (below 0, or 0.5 minutes or more late).

Limitations: the state is only kept while the page polls. If the first poll happens when the bus is already driving back and no bearing is available, that first reply still shows it; the next poll hides it. The heading rule has only two real samples behind it (a bus driving away at 151° from the stop direction, and an approaching bus at 94° on a curved street), so the 110° limit may need adjusting to about 120°.

A bug found during use. The first version of this check read a field (time) that does not exist in the data it worked on, so every bus counted as overdue. A bus that had just left the terminus could then be hidden as "moving away" even though it was not due for a while, and the page showed only timetable buses although the service had a live one. The check now uses min, and a test with realistically shaped data reproduces the old behaviour and passes with the fix. No further occurrence was reported after the fix.

## 4.8 The server clock

All arrival times come from the feed, so a web host whose clock is minutes off would make every "minutes left" wrong. After fetching the feed, bus.php compares its own clock with the HTTP Date header of Trafiklab's answer. A difference between 20 seconds and one hour is corrected for; smaller differences (cached responses, jitter) are ignored. The measured difference is shown as clock_offset_s in debug output, and the corrected time is returned as now_ts. The measured difference on the actual host was 0, so the host clock was correct. An earlier mismatch between the minutes and the clock time on the page, which was first suspected to be a clock error, therefore has another, unidentified cause; since then the page counts minutes from the clock time itself (section 6).

## 4.9 Caching and security

- Replies are cached for CACHE_SECONDS = 10 s in two files (with and without gps) in the temporary folder, to limit the number of Trafiklab calls. Each uncached request costs one call for TripUpdates and, with gps=1, a second one for VehiclePositions.

- The secret in ?t= protects the quota. It is visible to anyone who sees the page's requests, so it is a light protection, not strong security.

- The Trafiklab key exists only in bus_config.php. That file must not be shared or placed where it can be downloaded.

# 5. Response format

bus.php?t=SECRET returns the next buses; &gps=1 adds position data; &debug=1 lists more buses (up to 25) and extra fields, and shows hidden buses with a dropped note.

| Field | Meaning |
| --- | --- |
| now, now_ts | Server time HH:MM:SS (Stockholm) and as a Unix timestamp, from the corrected clock |
| src | live (from TripUpdates) or timetable (from the SL departures) |
| arrives | Predicted arrival at Sköntorp, HH:MM:SS (Stockholm time) |
| min | Minutes until arrival at the time of the request, one decimal; negative when overdue |
| delay_s | Delay in seconds from the feed (negative = ahead of schedule); null for timetable buses |
| dest, trip, route, dir_id | Destination (timetable only), trip ID (live only), and route and direction ids when the feed has them |
| lat, lon, where | With gps=1: position, and where with kind (at, between, near), the stop name(s), dist_m and stops_left |
| Debug only | clock_offset_s, feed_age_s, and per bus upd_age_s, pos_age_s, next_stop, stops_n, last_stop, rel, stop_rel, bearing, speed, vstop, vstatus, vseq, dropped |

Example (shortened):

{ "now": "16:05:44", "now_ts": 1792418744,

"buses": [

{ "src": "live", "arrives": "16:06:52", "min": 1.1, "delay_s": -68,

"lat": 59.29976, "lon": 18.05705,

"where": { "kind": "at", "stop": "Tämnarvägen", "dist_m": 60, "stops_left": 1 } },

{ "src": "timetable", "arrives": "16:34:00", "min": 28.3, "delay_s": null,

"dest": "Södersjukhuset" } ] }

# 6. The web page (bus.html)

- Access. The secret is put in the address once (bus.html#SECRET), stored in the browser's local storage and removed from the address bar. The page calls bus.php?t=…&gps=1 in the same folder.

- Refresh and pause. Every 20 seconds, for 3 minutes, and only while the tab is visible. Then the page pauses: the headline is replaced by a large yellow Pausad with "tryck för att uppdatera" and "(uppdaterad HH:MM:SS)", and the list is dimmed, so that old data cannot be mistaken for current data. A click on the headline (or Enter on the word) resumes, and the Uppdatera button still works. The pause protects the Trafiklab quota.

- Focus. The first bus of the server's list is the headline bus. There is no walking-time logic: the next bus only takes over when the server no longer lists the current one, which happens when the feed drops it or the GPS shows that it has passed.

- Countdown. Minutes left are computed every second from the arrival clock time and the browser's own clock (Stockholm time), so the big number always agrees with the time shown under it. If the browser's clock differs from the server's now_ts by 30 seconds or more, the page follows the server. The number is rounded down. With a GPS position, "Nu" is shown only when the bus is at Sköntorp; until then the smallest value is "Om 1 min", because the predicted time proved to be optimistic in the last stretch. Without a GPS position, less than a minute left gives "Nu". The delay text says "i tid" within ±60 s, otherwise the rounded minutes late or early. A badge shows Live or Tidtabell.

- Position row. A small SVG bus replaces the words: "Årsta torg [bus]" when the bus is at a stop, "Årsta torg [bus] Årstaskolan" between two stops. Each name shortens with "…" by itself on a narrow screen, so both stay visible. The full sentence remains as hover text. The stops-left count is on its own row.

- Fixed layout. The headline always has the same rows (countdown, time and delay, position, stops left) and the list always has three rows, filled with empty ones if needed, so that the page keeps its height.

- Errors. The page shows the HTTP status and the first words of the reply when the server does not return JSON. If a later request fails while earlier data exist, the old data stay on screen without a message (a known weakness).

stop_display.html is a separate page that mimics the display at the stop, with both directions, line badge, destination, minutes and clock time. It calls the SL API directly (two calls, direction=1 and direction=2). Whether the browser may call the SL API from a web page was not tested; sl_proxy.php is the fallback.

# 7. The ESP32 display (bus_display.ino)

## 7.1 Hardware and software environment

The gadget is a Waveshare ESP32-C6-Touch-LCD-1.54 (sold by Electrokit): ESP32-C6, 1.54 inch 240×240 touch screen, 16 MB flash, LiPo charging circuit. The pins below come from Waveshare's documentation.

| Function | Pins and details |
| --- | --- |
| Screen (ST7789, SPI) | SCK 1, MOSI 2, DC 3, CS 11, backlight 6; reset on GPIO4, which is also the reset of the touch chip |
| Touch (CST816, I2C) | Address 0x15, SDA 8, SCL 7, interrupt (INT) on GPIO5; INT idles high and goes low on touch |
| Buttons | BOOT on GPIO9, PLUS on GPIO18. The PWR button is a power switch, not readable by the code. |
| SD card | Chip select GPIO17; the card shares the SPI lines with the screen. Not used. |
| Audio amplifier | Enable on GPIO15 (kept off) |
| Battery | Divider on GPIO0: battery voltage = ADC voltage × 3 |
| USB | The chip's own USB (serial over USB with "USB CDC On Boot") |

Arduino IDE settings: board "ESP32C6 Dev Module", USB CDC On Boot "Enabled", Flash Size 16MB, Partition Scheme "Default 4MB with spiffs", esp32 board package 3.3.x. Libraries: GFX Library for Arduino, U8g2, ArduinoJson (version 6 and 7 both compile; version 7 gives a harmless warning about StaticJsonDocument). The private settings are in config.h, in the same folder as the sketch.

## 7.2 What happens in one cycle

- The screen is off and the chip is in deep sleep.

- A touch pulls the touch chip's interrupt pin (GPIO5) low, which wakes the chip.

- Start-up: release the pin holds from the sleep, switch the amplifier off and deselect the SD card, start the display (this also resets the touch chip), draw "Hämtar…" and switch the backlight on.

- Join Wi-Fi: first with the channel and access point saved in RTC memory (well under a second), otherwise by scanning.

- Request https://HOST/PATH?t=SECRET&gps=1 and read the JSON.

- Draw the screen. Every 250 ms only the parts that changed are redrawn; new data is fetched every 20 seconds.

- A touch or a button press while awake refreshes at once and restarts the minute.

- After 60 seconds: Wi-Fi off, screen controller in sleep mode, backlight off, pins held, deep sleep.

## 7.3 Sleeping and waking

Why only touch can wake it. The ESP32-C6 can wake from deep sleep only on GPIO0 to 7 (Espressif's documentation). The touch interrupt (GPIO5) is within that range; the BOOT (GPIO9) and PLUS (GPIO18) buttons are not, so they cannot wake the chip from deep sleep. The wake-up is configured as "EXT1, any low", because the pin idles high.

What the firmware does to make the touch wake-up work and to keep the sleep current low:

- Pin holds. Before sleeping, GPIO4 (reset of screen and touch chip) is held high, the backlight low, the screen and SD chip selects high and the amplifier enable low. On the ESP32-C6 each pin is held with gpio_hold_en(); the function gpio_deep_sleep_hold_en() used on older chips does not exist for it. After wake-up each pin is first set to the level it was holding, and only then released, so that no glitch resets the touch chip.

- Finger release. The firmware waits until the touch interrupt is high again (a finger on the glass would wake it straight away) and reads the touch chip once to clear a pending touch.

- Safety. If the interrupt is still low after 3 seconds, the chip sleeps with a 5-minute timer instead of the touch wake-up, to avoid a wake-up loop that would empty the battery.

- Screen controller. The ST7789 is put into "sleep in" mode, which draws far less current than only switching the backlight off.

- Light-sleep alternative. USE_DEEP_SLEEP 0 in config.h selects light sleep, which wakes on touch and on the BOOT and PLUS buttons and keeps the memory; it is the fallback if the touch wake-up would not work.

- Wake-up letter. A small letter in the bottom right corner shows why the chip woke: T = touch, P = power on or restart, L = light sleep, ? = other (turned off with SHOW_WAKE_INFO 0).

Not controlled by the firmware: the audio chips, the motion sensor, the charger and the voltage regulators of the board also draw current while the chip sleeps. The sleep current has not been measured, and the battery life is therefore not known (section 9).

## 7.4 Time and numbers

The gadget needs no clock or time zone of its own. The JSON contains the server's time of day (now, Stockholm time) and each bus's arrival time of day (arrives). The firmware stores the server time at the moment of the request (the middle of the request, since the server read its clock about half-way through) and adds the time that has passed since. Minutes left = arrival − that time. The rules from the web page apply: with a GPS position "Nu" only when the bus is at Sköntorp, otherwise at least "Om 1 min"; the delay text and the abbreviations are the same. Battery percentage: voltage between 3.3 V (0 %) and 4.15 V (100 %) on a straight line, an approximation of a LiPo curve.

## 7.5 The screen

The screen is divided into regions. Each region has a signature of what it shows, and it is only cleared and redrawn when the signature changes, so nothing flickers.

| Region | Content | Colour |
| --- | --- | --- |
| Header | "Buss 168 mot Sös" and battery percentage | White (battery red below 20 %) |
| Countdown | "Om 4 min", "Nu", or "Hämtar…" / "Inga avgångar" / "Fel" | Green = live, amber = timetable; white for messages, red for errors |
| Time and delay | "13:39 · i tid" | Same colour as the countdown |
| Position | Name, bus icon and, between two stops, the second name | Green |
| Stops left | "3 Hpl kvar" or "Nästa Hpl är din" | Green |
| List | The next three buses: clock time, countdown, dot (filled = live, ring = timetable) | Green or amber per row |
| Footer | "Uppdaterad hh:mm" and the wake-up letter | Dim grey |

- Swedish letters. The text uses U8g2 fonts of the _tf kind (helvB24, helvB14, helvB12, helvR14, 6x13), which contain the whole Latin-1 set, with UTF-8 printing switched on. The arrow and the en dash are not in Latin-1 and are avoided.

- Abbreviations. "Södersjukhuset" is written "Sös" and "hållplats" "Hpl".

- Bus icon. Drawn in code from rounded rectangles and circles, in the colour of the row.

- Long names. The names are measured with the font, and the longer one is shortened one character at a time (whole UTF-8 characters) with a dot at the end until both fit next to the icon.

- Colours. Green 0x07E0 (live), amber 0xFD80 = #FFB000 (timetable), white, dim grey 0x8410 and red 0xF800 (RGB565). The dots in the list keep the live/timetable difference visible also for people who cannot tell green from amber.

## 7.6 Settings and security

config.h holds the Wi-Fi name and password, the address and path of bus.php, and the secret. It is a separate file, like bus_config.php on the server, and must not be shared. The connection is encrypted (HTTPS) but the server's certificate is not checked (setInsecure). That is acceptable for a private gadget on your own network, but it is a shortcut.

## 7.7 How it was built and verified

- A first sketch written with another AI assistant showed the buses but used light sleep with button wake-up; deep sleep and the Swedish letters had failed there. The present firmware replaced it and reuses the screen set-up and the data handling that worked.

- The first version of bus_display.ino did not compile in the Arduino IDE, for two reasons. (1) The IDE inserts the prototypes of all functions in front of the first function, so a type used in a function signature (Region) must be defined above it. (2) gpio_deep_sleep_hold_en() does not exist for the ESP32-C6. Both were corrected.

- The corrected sketch was compiled and linked with the real toolchain (Arduino-ESP32 3.3.12, RISC-V GCC 14.2) against ArduinoJson 6.21.5 and 7.4.2. The same set-up reproduced the two errors above exactly before the fix. The program size is estimated at about 1.25 MB of the 1.31 MB in the partition "Default 4MB with spiffs": it fits, but with little room. If the IDE reports "sketch too big", use the partition scheme "8M with spiffs (3MB APP/1.5MB SPIFFS)", which the 16 MB flash allows.

- The logic (parsing, countdown, "Nu" rule, layout texts) was also run on a computer against sample JSON. The behaviour on the board was verified by you: the firmware was uploaded and shows the buses.

# 8. Accuracy: measurements

## 8.1 Method

bus_log.py logs, every 30 seconds, the predicted arrival at Sköntorp from the SL API and from the TripUpdates feed, plus the GPS position of the approaching bus. When a bus reached the stop, the arrival was recorded by hand with bus_log.py mark. The report compares the predictions made 10, 5 and 2 minutes earlier with the marked time. Error = predicted − actual; a positive value means that the bus arrived earlier than predicted. Values are in seconds; "n/a" means that nothing was logged near that time.

## 8.2 Results for six buses

| Bus arrived | API 10 | API 5 | API 2 | Live 10 | Live 5 | Live 2 |
| --- | --- | --- | --- | --- | --- | --- |
| 16 Oct 08:47:08 | +52 | +52 | +52 | −162 | −100 | −20 |
| 16 Oct 10:59:09 | +291 | +291 | +291 | +137 | +137 | −120 |
| 16 Oct 16:53:10 | n/a | −70 | −10 | n/a | −116 | −60 |
| 17 Oct 09:14:56 | +64 | +120 | +64 | +57 | +120 | −7 |
| 17 Oct 09:52:16 | +92 | +44 | +44 | −49 | +14 | −64 |
| 17 Oct 10:22:30 | +30 | +120 | +30 | +33 | +90 | +3 |

| Minutes before | Buses | Mean ¦error¦ API | Mean ¦error¦ live | Worst API | Worst live | Mean error API | Mean error live |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 5 | 106 s | 88 s | +291 s | +137 s | +106 s | +3 s |
| 5 | 6 | 116 s | 96 s | +291 s | +137 s | +93 s | +24 s |
| 2 | 6 | 82 s | 46 s | +291 s | +3 s | +79 s | −45 s |

- The live feed had the smaller average error at all three checkpoints, most clearly in the last two minutes, where it was never more than 3 seconds too late and on average slightly early.

- The API was on average too late by roughly 80 to 105 seconds (the bus came earlier than shown), with one case of almost 5 minutes.

- Five to ten minutes ahead, both sources can be off by more than two minutes; the live feed erred in both directions. The worst case for the live feed was a bus arriving 137 seconds before the prediction made 5 minutes earlier.

Caveats: six buses is a small sample, the arrival was marked by hand, which adds some uncertainty (for the bus at 10:59 the GPS put it at the stop about two minutes before the mark), and most buses are from morning and midday (one afternoon bus, no evening buses). The measurements were not repeated after the later changes to the page and the gadget.

## 8.3 GPS distance versus time, and why "Nu" waits for the GPS

analyze_gps.py relates the straight-line distance from the stop to the minutes before arrival. Two of the three first buses were about 500 m away for several minutes (for example 443 to 497 m from 6 to 3 minutes before, and 499 to 502 m from 6 to 4 minutes before), so distance alone cannot be turned into a reliable time. This is why the GPS is used to say where the bus is, while the arrival time still comes from the feed.

A later debug sample shows why "Nu" is tied to the GPS: at 09:16:22 the bus was at the stop before Sköntorp, about 290 m from Sköntorp in a straight line and moving at 4.2 m/s, while the prediction said 37 seconds. At that speed the bus needs about 70 seconds even on a straight line, and the street is longer. Showing "Nu" from the prediction alone came too early, about two stops before the bus arrived.

# 9. What has and has not been verified

| Item | Status |
| --- | --- |
| Position text compared with the SL app | Checked by you: accurate once the route list contained all stops. The wrong "between" text at bends was reported twice and fixed (4.6); the fix has not been reported on yet. |
| Direction guard | Checked by you: only buses toward Södersjukhuset are shown |
| Hiding a bus that has passed | Debug output showed has left the stop; the later bug fix was not reported as a problem again |
| Arrival accuracy | Six manual measurements (8.2) |
| Server clock | Measured difference 0 s on your host (4.8) |
| SL timestamps | One debug sample (3.2) |
| ESP32 sketch compiles and links | Yes, with the real toolchain for ESP32-C6 (7.7) |
| ESP32 firmware on the board | Reported working by you (uploaded, shows the buses, in use); there is no formal test record of the deep-sleep touch wake-up |
| Battery life of one month | Not known: the test started in late October 2026 and the result is pending. The sleep current has not been measured. |
| Calling the SL API directly from a browser | Not tested (CORS unknown) |
| Trafiklab quota figures | From memory; check Trafiklab |

# 10. Known limitations and open points

- Both sources are uncertain more than a few minutes ahead. A safety margin at the stop is advisable, since buses were seen arriving up to nearly five minutes before the SL API time.

- The static data are needed to produce LAST_STOP_IDS and ROUTE_STOPS. Stop IDs are stable, trip IDs are not, so the trip list in bus_cache.json must be refreshed now and then if bus_log.py is used. If SL changes the stop IDs or the route, the settings must be regenerated.

- The passed-bus logic uses straight-line distances and a few thresholds from a small number of observations. It remembers a bus only while someone polls the server. A gadget that wakes once per touch gives the server no history, so for it only the heading rule works, and the first reply after a bus has just left may still show that bus.

- The position text is based on straight lines between stops and can be wrong on winding stretches of the route.

- The battery percentage is a straight-line approximation of a LiPo curve, and it is not meaningful while the board is charging over USB.

- The gadget does not check the server's certificate, and the secret travels in the address of the request.

- The Trafiklab calls scale with use: each uncached request costs one or two calls.

- The web page does not tell the user when a refresh has failed and old data are shown.

- The program is close to the size limit of the default partition (7.7).

# 11. Setup and maintenance

## 11.1 Server and Mac

- Create a Trafiklab project with two keys: one for GTFS Regional Static data and one for GTFS Regional Realtime data.

- On the Mac, in a Python virtualenv: python3 -m pip install gtfs-realtime-bindings; set TRAFIKLAB_STATIC_KEY and TRAFIKLAB_RT_KEY in the terminal.

- python bus_log.py prepare downloads the static timetable once and writes bus_cache.json.

- python check_trip.py prints LAST_STOP_IDS; python make_route_stops.py prints ROUTE_STOPS (raise its BEFORE setting so that the whole route is included).

- Fill in bus_config.php (key, secret, LAST_STOP_IDS, ROUTE_STOPS) and upload bus.php, bus_config.php and bus.html to the same folder.

- Open bus.html#SECRET once in the browser.

## 11.2 The gadget

- Install the Arduino IDE, add the esp32 board package (3.3.x) and the libraries GFX Library for Arduino, U8g2 and ArduinoJson.

- Make a sketch folder with bus_display.ino and config.h (the folder and the .ino file must have the same name). Fill in config.h.

- Choose the settings in 7.1, connect the board with a USB-C data cable and upload. If the board does not connect, hold BOOT while plugging in the cable.

- After about a minute the screen switches off. Touch it: it should wake with the letter T in the bottom right corner.

## 11.3 Settings in bus_config.php (defaults in bus.php)

| Setting | Default | Meaning |
| --- | --- | --- |
| RT_KEY | none | Trafiklab realtime key |
| SECRET | none (required) | Value the page and the gadget send as ?t= |
| STOP_IDS | 9022001013119001 | Sköntorp stop ID in the realtime feed |
| LAST_STOP_IDS | empty | End stops of the right trips; empty = no direction guard |
| ROUTE_STOPS | empty | Ordered stops of the route; empty = no position text |
| SITE_ID, DIRECTION, LINE, DEST_MATCH | 1512, 2, 168, Södersjukhuset | Selection of timetable buses |
| AT_STOP_M, FAR_M | 120 m, 2000 m | "At the stop" radius; limit for giving no text |
| MATCH_SECONDS | 480 | Live and timetable closer than this = same bus |
| LIVE_GRACE_S | 300 | How long a late live bus stays listed |
| MOVING_AWAY_M | 80 | Margin for the "moving away" rule |
| CACHE_SECONDS | 10 | Reply cache |
| ROUTE_IDS, DIRECTION_ID | empty, null | Filters on route and direction ids, not usable while SL leaves them empty |

## 11.4 Settings in config.h (gadget)

| Setting | Default | Meaning |
| --- | --- | --- |
| WIFI_SSID, WIFI_PASSWORD | none | Your Wi-Fi (2.4 GHz) |
| BUS_HOST, BUS_PATH | none | Address of bus.php without "https://", and the path to it |
| BUS_SECRET | none | The same secret as SECRET in bus_config.php |
| AWAKE_SECONDS | 60 | How long the screen stays on after a touch |
| REFRESH_SECONDS | 20 | How often new data is fetched while the screen is on |
| USE_DEEP_SLEEP | 1 | 1 = deep sleep, wakes on touch; 0 = light sleep, wakes on touch or buttons |
| SHOW_WAKE_INFO | 1 | Show the wake-up letter in the corner |

## 11.5 Troubleshooting

- Open bus.php?t=SECRET&gps=1&debug=1. A bus with a dropped field exists but is hidden, and the field says which rule hid it. clock_offset_s shows the host clock difference.

- HTTP 403 from Trafiklab: wrong or missing key, or the key lacks the API; check that the right key is used for the right feed.

- "not JSON" on the page: the address of bus.php is wrong (a "not found" page was returned) or the host blocks the request.

- Gadget shows "Fel": the text under it says what failed (for example "WiFi saknas", "HTTP 403", "JSON-fel").

- Gadget does not wake on touch: set USE_DEEP_SLEEP 0 and report what happens. During deep sleep the USB port disappears; hold BOOT while plugging in the cable to upload.

- "Sketch too big" when uploading: choose the partition scheme "8M with spiffs (3MB APP/1.5MB SPIFFS)".

- To check a trip or a stop, run check_trip.py with the trip ID; it prints line, direction, first and last stop and the scheduled time at Sköntorp.